CENICA MÉXICO

VEHICLE EMISSIONS EVALUATION

Results Delivery

| Date | 19-Dec-05 | Type of Test | 505-HOT |
| --- | --- | --- | --- |
| Organism | Bio Plus Fuel | Additive/Apparatus | YES |
| Vehicle Type | VW | Catalitic Converter | NO |
| Year Model | 1995 Sedan | Combustible | Gasoline |
| No. of licence | Non | Inertial mass (lb) | 2000 |
| Motor (liters) | 1.6 | Road cargo (hp) | 77 (@ 50 mph) |
| Transmission | MANUAL | Equipment operator | Ing. Alejandro García F. |
| Vehicular weight (kgs) | 820 | Vehicle operator | Tec. Francisco Guardado L. |
| P. B. V. |  | Observations |  |
| Odometer | 60423 |  |  |

PARAMETERS

Cycle 505

| Barometric pressure (mmHg) | 505 |
| --- | --- |
| Temperature (°C) | 17 |
| Vapor pressure (mmHg) | 14.53 |
| Revolutions | 10699 |
| Mix Vol. | 2436.96 |
| Distance (km) | 5692 |
| Relative Humidity (%) | 42 |
| Factor DT | 17.2 |

|  | LECTURES | PARTIAL RESULTS |
| --- | --- | --- |

|  |  | Sample | Room |  | (grams) | (g/km) |
| --- | --- | --- | --- | --- | --- | --- |
| CO |  | 1342 | 49 |  | 147.76 | 24.98 |
| CO2 |  | 78 | 5 |  | 721.05 | 122.59 |
| HC |  | 1240 | 23 |  | 14.5 | 2.45 |
| NOx |  | 718 | 21 |  | 6.11 | 1.38 |

FINAL RESULTS

| CO |  | CO2 |  | HC |  | NOX |  | PERF. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g/Km |  | g/Km |  | g/Km |  | g/Km |  | Km/l |
| 24.98 |  | 122.89 |  | 2.48 |  | 1.38 |  | 14.08 |

Done by				Verified by				Vo. Bo.

Tec. Francisco Guardado López	      Ing. Alejandro García Fragoso

Lab. de Emisiones Vehiculares	      Evaluación de Emisiones Vehiculares