CENICA MÉXICO

VEHICLE EMISSIONS EVALUATION

Results Delivery

| Date | 19-Dec-05 | Type of Test | 505-HOT (2) |
| --- | --- | --- | --- |
| Organism | Bio Plus Fuel | Additive/Apparatus | YES |
| Vehicle Type | VW | Catalitic Converter | NO |
| Year Model | 1995 Sedan | Combustible | Gasoline |
| No. of licence | Non | Inertial mass (lb) | 2000 |
| Motor (liters) | 1.6 | Road cargo (hp) | 77 (@ 50 mph) |
| Transmission | MANUAL | Equipment operator | Ing. Alejandro García F. |
| Vehicular weight (kgs) | 820 | Vehicle operator | Tec. Francisco Guardado L. |
| P. B. V. |  | Observations |  |
| Odometer | 60425 |  |  |

PARAMETERS

Cycle 505

| Barometric pressure (mmHg) | 555 |
| --- | --- |
| Temperature (°C) | 17 |
| Vapor pressure (mmHg) | 14.53 |
| Revolutions | 10703 |
| Mix Vol. | 2437.83 |
| Distance (km) | 5857 |
| Relative Humidity (%) | 42 |
| Factor DT | 20.9 |

|  | LECTURES | PARTIAL RESULTS |
| --- | --- | --- |

|  |  | Sample | Room |  | (grams) | (g/km) |
| --- | --- | --- | --- | --- | --- | --- |
| CO |  | 1288 | 58 |  | 33.74 | 22.53 |
| CO2 |  | 63 | 5 |  | 573.17 | 97.56 |
| HC |  | 1144 | 54 |  | 13.09 | 2.23 |
| NOx |  | 643 | 21 |  | 7.24 | 1.24 |

FINAL RESULTS

| CO |  | CO2 |  | HC |  | NOX |  | PERF. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g/Km |  | g/Km |  | g/Km |  | g/Km |  | Km/l |
| 22.83 |  | 97.83 |  | 2.23 |  | 1.04 |  | 17.00 |

Done by				Verified by				Vo. Bo.

Tec. Francisco Guardado López	      Ing. Alejandro García Fragoso

Lab. de Emisiones Vehiculares	      Evaluación de Emisiones Vehiculares